Dear all,

We talked about the software, and here below are critical functions that the software shall cover, for that I would like to have your verification and comments before our next meeting.

Attached you will find the TQI calculation for Taipei Metro.

Please feel free to contact me via Teams for any questions.

Bai

Software Functions System by System

TGMS / RP

- TQI

It is necessary to calculate Track Quality Index (TQI), and the software shall allow operator to define the formular and basic value.

- TQI calculation

  - Basic Value - Allow operator to define

  - Calculation Formular

  - TQI Segmentation

    - Tangent Line

    - Curve

    - Transition Curves

- GIS database

  - The software Allows Operator to establish a GIS database as per starting km to ending km.

  - Allows Operator to establish characteristics / Parameters for GIS data base with classified symbols and colors, such as

    - Sleeper Types

    - Slab

    - Ballast

    - Curvatures

    - Tonnage

    - Switches

- Track Segmentation

  - Track Segmentation will be proceeded according to the above characteristics defined and selected by operators. For example, segmentation of all curves with curvature of 200 meters.

  - Track Segmentation can be proceeded according to starting km to ending km.

  - Track Segmentation can be proceeded according to a defined length, such as 5 km, as per operator.

- Maintenance Management

  - Allow operator to establish maintenance thresholds

    - Safety Level T3

    - Maintenance Level T2

    - Management Level T1

  - Visualize the results as % percentage of km for a parameter plot over "T2"

  - Visualize the result of percentage plot for a section of track / segmentation selected by operators for multi-diagnostic data over a period, such as three month.

  - Allow operator to set up maintenance rules based on thresholds and segmentations.

    - Maintenance policy = "Change Rail" if "Gauge" > T3 and W4 > T3

    - Maintenance Policy = "Grinding" if W2 > T2

- Database of Maintenance Records

  - The software shall allow operator to build up maintenance records in the database.

  - Allow to search at certain period the records, such as "change rail", "Grinding"

  - Visualize the records by symbols at measurement / statics plot.

- Data Validation

  - Allow to align the parameter plot of multi acquisitions as per km.

  - Allow to delete the parameter csv data manually base on starting km to ending km if the data is not correct.

- Report / PDF

  - Generate report for all the analysis.

  - Generate a database for all information of analysis done by operator.

- Interface with ERP system of Client